Name: Jazz_sbc18h2_v3.1_ADS2011

Version: 3.1

Date: 06 January 2012

ADS Version: ADS 2009 and above

Description:

ADS front-to-back Design Kit for the:

Jazz 0.18u SiGe SBC18H2 PROCESS

Components and models are based on the following Jazz model files:

| Process | Version |
| --- | --- |
| 0.18u SiGe (SiGe200) | v6.8_sbc18h2 |

This release note describes the features of ADS design kit for Jazz SBC18H2 PROCESS model version 6.8 for nominal, statistical and x-sigma process.

Please note –

- Gate connection (unconnected, top, bottom, both, alternate) and source/drain metal connection styles (none, metal1, metal2) have been added to RF FETs (nfet_rf, nfet3p3_rf, pfet_rf, pfet3p3_rf).

- This release includes the CNEX component definitions for CDL export.

- For x-sigma corner case, value of sigma should be mentioned in the process component.

- The value of ‘m’ for capacitors has been taken as 1 by default.

- WARNING message for the ‘polyimide’ parameter in inductors l3t, l3t_oct, ldiff and ldiff_oct, you must manually verify the following when using the Polyimide option:

  - Verify all inductors in the design use the same setting for this option.

  - Verify the Jazz process variant used will support Polyimide.

  - Select the Polyimide option during tape-out submission to Jazz.

- Components pdiode and pdiode3p3 have been included for legacy purpose only.

- The cdsio.v4 inductor tool used in this kit is not supported on  Windows platform. It can be used with SUNOS and LINUX.

- Every time on changing the inductor parameters, the ‘update’ option should be changed to ‘yes’ for the change to take place.

- For using the kit in LINUX and SUN, 4 environment variables need to be set as described in Jazz_sbc18h2_install.doc. Only after setting up these variables, start ADS for the inductor components to work properly. If ADS is already running when you set these variables, restart ADS for the inductor components to work properly.

- The palette in which the component is present is mentioned along the component name below.

- The palette in which the component is present is mentioned along the component name below.

- Devices with geometric parameters now only accept on-grid values i.e. in multiples of 0.005 um.

- There is a rules file in the “drc” directory of the kit to check for off-grid errors in the layout components.

Table of Contents

- PROCESS

- PNP TRANSISTORS

- NPN TRANSISTORS

- RESISTORS

- DUMMY METAL RESISTORS

- CAPACITORS

- VARACTORS

- FETS

- INDUCTORS

- DIODES

- FUSE

- ESD DIODES

- OTHER ESD COMPONENTS

  - Process (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_Process | NOMINAL model, FAST model, SLOW Model, FASTSLOW model, SLOWFAST model, STATISTICAL model, X-SIGMA model |

  - PNP Transistors (Jazz sbc18h2 v2 Active)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_vpnp | pnpa , pnpb, pnpc ,pnpd |

  - NPN Transistors (Jazz sbc18h2 v2 Active)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_npn | npn transistors |

  - FETS (Jazz sbc18h2 v2 Active)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_nfet | nfet, n3p3fet |
| Jazz_sbc18h2_nfet_4 | nfet, n3p3fet (with explicit vss pin) |
| Jazz_sbc18h2_nfet_rf | nfet_rf, n3p3fet_rf |
| Jazz_sbc18h2_nfet_esd | nfet_esd, n3p3fet_esd |
| Jazz_sbc18h2_pfet | pfet, p3p3fet |
| Jazz_sbc18h2_pfet_4 | pfet, p3p3fet (with explicit vdd pin) |
| Jazz_sbc18h2_pfet_rf | pfet_rf, p3p3fet_rf |

  - RESISTORS (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_polyres | rpp3t,rph3t |
| Jazz_sbc18h2_rnwell | rw3t |
| Jazz_sbc18h2_rtin | rtin |
| Jazz_sbc18h2_rps | rps |
| Jazz_sbc18h2_r_dummy | Rmet1, rmet2, rmet3, rmet4, rmet5, rmet6 |

  - CAPACITORS (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_mimcap3t | cmim2 |
| Jazz_sbc18h2_mimcap4t | cmimw2_4 |
| Jazz_sbc18h2_polycap | pc, pc3p3 |

  - VARACTORS (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_varactor_mos | varactor_mos |

  - INDUCTORS (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_ind | l3t |
| Jazz_sbc18h2_ind_diff | ldiff |
| Jazz_sbc18h2_ind_oct | l3t_oct |
| Jazz_sbc18h2_ind_diff_oct | ldiff_oct |

  - DIODES (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_dio | ndiode, n3p3diode, pdiode, p3p3diode |
| Jazz_sbc18h2_pdiode_3t | pdiode_3t, p3p3diode_3t |
| Jazz_sbc18h2_dnw | diso, ddnw |
| Jazz_sbc18h2_dnwell | nwdiode |

  - FUSE (Jazz sbc18h2 v2 Passive)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_fuse | fuse |

  - ESD DIODES (Jazz sbc18h2 v2 ESD)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_esd_diode_sub | esd_diode_sub_p5x8_1 |
|  | esd_diode_sub_p5x8_2 |
|  | esd_diode_sub_p5x8_3 |
|  | esd_diode_sub_p5x8_4 |
|  | esd_diode_sub_p5x8_6 |
|  | esd_diode_sub_p5x8_8 |
| Jazz_sbc18h2_esd_diode_well | esd_diode_well_p5x8_1 |
|  | esd_diode_well_p5x8_2 |
|  | esd_diode_well _p5x8_3 |
|  | esd_diode_well _p5x8_4 |
|  | esd_diode_well _p5x8_6 |
|  | esd_diode_well _p5x8_8 |

  - OTHER ESD COMPONENTS (Jazz sbc18h2 v2 ESD)

| Component Name | Components available as subtype |
| --- | --- |
| Jazz_sbc18h2_pad_no_esda | pad_no_esda |
| Jazz_sbc18h2_bigfet_3p3 | bigfet_3p3 |

Known issues:

Following are the known issues -

- For ADS 2011 and onwards, bulk pins disappear on modifying the bulk properties for the components which have the default bulk setting as “n” or “NONE”. To overcome this, in such components, place the component with the bulk present at least on one side i.e. not “n” or “NONE”.

- On Windows platform, the inductors ind, ind_oct, ind_diff and ind_diff_oct simulate correctly only for the default values of the parameters.
The inductor simulation functionality is fully supported only on the UNIX platform.

- This PDK cannot be used with ADS2006A UR2 and UR3. On doing this, the error received is –

Error detected by hpeesofsim during netlist parsing.

Syntax error in `hByBaseWidth_gp_dw' instance statement.

- If Visual Studio is installed in any directory other than C drive, none of the components simulate in WIN Vista 64 bit.

This error is given below –

Warning detected by hpeesofsim in Verilog-A based device loading during netlist parsing.

Load of dynamic code failed for file 'd:\users\default08\hpeesof\agilent-model-      cache\cml\1.42\win32_64/models_03440_20080212_140429_000041/lib.win32_64/rnw.cml'.

Dynamic linker gave reason:

"The specified module could not be found."

Error detected by hpeesofsim in loading Verilog-A device during netlist parsing.

'#load' command, failed to load the Verilog-A CML file :

`d:\users\default08\hpeesof\agilent-model-cache\cml\1.42\win32_64/models_03440_20080212_140429_000041/lib.win32_64/rnw.cml'

Following types of warning messages are displayed during simulation –

1. Warning: parameter 'subs' for BJT model `pnpa' is not currently supported.

Warning: parameter 'level' for BJT model `pnpa' is not currently supported.

Warning: parameter 'subs' for BJT model `pnpb' is not currently supported.

Warning: parameter 'level' for BJT model `pnpb' is not currently supported.

Warning: parameter 'subs' for BJT model `pnpc' is not currently supported.

Warning: parameter 'level' for BJT model `pnpc' is not currently supported.

Warning: parameter 'subs' for BJT model `pnpd' is not currently supported.

Warning: parameter 'level' for BJT model `pnpd' is not currently supported.

2. Warning detected by hpeesofsim in topology check during circuit set up.

Number of nodes with no DC path to ground (topology corrected): 2

(The no. of nodes mentioned may vary in the warning messages)

3. Warning detected by hpeesofsim in M1.mds during circuit set up.

Noff = 4.4159 is too large.

Warning detected by hpeesofsim in M1.n3p3fet during circuit set up.

nsub is ignored because k1 or k2 is given.

Warning detected by hpeesofsim in topology check during circuit set up.

Number of nodes with only one device attached (topology corrected): 2

Warning detected by hpeesofsim in M1.M0 during circuit set up.

Noff = 4.4159 is too large.

Warning detected by hpeesofsim in n3p3fet during circuit set up.

nsub is ignored because k1 or k2 is given.

Warning detected by hpeesofsim in topology check during circuit set up.

Number of nodes with only one device attached (topology corrected): 2

(Similar warnings maybe displayed for other mosfets as well)

4. Warning detected by hpeesofsim in C1.mpc during circuit set up.

Noff = 5.938 is too large.

Warning detected by hpeesofsim in C1.mpc during circuit set up.

Voffcv = 99.905 is too large.

Warning detected by hpeesofsim in C1.mpc during circuit set up.

Moin = 3 is too small.

Warning detected by hpeesofsim in pc_fet during circuit set up.

nsub is ignored because k1 or k2 is given.

Warning detected by hpeesofsim in C1.mpc during circuit set up.

Noff = 6.5 is too large.

Warning detected by hpeesofsim in C1.mpc during circuit set up.

Voffcv = 199.85 is too large.

Warning detected by hpeesofsim in C1.mv1p8 during circuit set up.

nsub is ignored because k1 or k2 is given.

Warning detected by hpeesofsim in C1.qin during circuit set up.

The parasitic resistor Rc = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in C1.qin during circuit set up.

The parasitic resistor Re = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

(In the error, the value of Noff, Voffcv and Moin may vary from above. Similar warnings may be seen in other capacitors and varactors as well.)

5. Warning detected by hpeesofsim in Q1.q3 during circuit set up.

The parasitic resistor Rc = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in Q1.q3 during circuit set up.

The parasitic resistor Rb = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in Q1.q3 during circuit set up.

The parasitic resistor Re = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

6. Warning detected by hpeesofsim in I1.Q1.qin during circuit set up.

The parasitic resistor Rc = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in I1.Q1.qin during circuit set up.

The parasitic resistor Re = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in I1.M12.p3p3fet during circuit set up.

nsub is ignored because k1 or k2 is given.

Warning detected by hpeesofsim in I1.M15.mds during circuit set up.

Noff = 4.4159 is too large.

Warning detected by hpeesofsim in topology check during circuit set up.

Number of nodes with only one device attached (topology corrected): 1

Number of nodes with no DC path to ground (topology corrected): 2

Warning detected by hpeesofsim in I1.Q0.qin during circuit set up.

The parasitic resistor Rc = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in I1.Q0.qin during circuit set up.

The parasitic resistor Re = 0 (after area and multiplicity scaling)

was smaller than or equal to Minr, and has been deleted.

Warning detected by hpeesofsim in I1.M0.p3p3fet during circuit set up.

nsub is ignored because k1 or k2 is given.

Warning detected by hpeesofsim in I1.M1.mds during circuit set up.

Noff = 4.4159 is too large.

(Similar errors may be seen for ESD components).