Design Kit Installation Procedure

Name: Jazz_sbc18h2_v3.1_ADS2011

Version: 3.1

Date: January 06 2012

# Pre-requisite

The following procedure requires ADS 2009 and above.

## Kit Installation (for versions previous to ADS 2011.01)

- In the ADS main window select DesignKit>Install Design Kits...

- Unzip the Jazz_sbc18h2_v3.1_ASD2011.zip archive in a directory on your hard drive (<your_dir>) using the Unzip Design Kit Now Button in the Install ADS Design Kit dialog. The design kit directories and files will then be inflated into <your_dir>\ Jazz_sbc18h2_v3.1_ADS2011\...

- Verify the information in the Install ADS Design Kit dialog including the installation level and click OK.

Note: At this point the design kit is loaded into ADS; however the Jazz sbc18h2 v6.8 models still need to be installed to simulate the components.

## Kit Installation (for versions ADS 2011.01 and after)

- If you intend to use the kit in a new workspace, then take the following steps:

  - Start the “New Workspace Wizard”. Hit Next.

  - Fill the relevant fields on the “Workspace Name” page. Hit Next.

  - On the “Add Libraries” page, add the PDK using the “Add User Favorite Library/PDK…” dialog (browse to the kit location and select lib.defs). Hit Next.

  - Name of a work-library, to be created inside the workspace. Hit Next.

  - On the “Technology” page, choose the kit you intend to use and attach it with the work-library. Hit Next

  - Review the workspace summary. Hit Finish if done.

The workspace and the work-library with the design kit loaded and its technology attached is now ready for use.

- If you intend to use the kit in an already opened workspace, then take the following steps:

  - Create a new library (recommended). You will be presented with a “New Library Wizard”, similar to the “New Workspace Wizard”. In the wizard, name the work-library and attach the technology as defined in 1.d and 1.e above. The new library is ready for use.

A sample workspace should appear as in the screenshot below

## Installing the device models (separate from the kit)

- This design kit supports following JAZZ models:

| Process | Version |
| --- | --- |
| 0.18u SiGe (SiGe200) | v6.8 |

This design kit is setup to work only with the specified versions of the model files specified here. If new model versions become available, a new version of the ADS design kit will be made available.

The models are not contained in the kit and are to be separately procured from the foundry.

This kit supports the “spectre” models for nominal, fast, slow, slowfast, fastslow corners and also for statistical and x-sigma simulations.

- You may get the models in an archived format with a file name similar to sbc18h2_model_v6p8.tar.gz
After unzipping the above mentioned file, the model files will be present in the directory amslibs/cds_default/cdslibs/sbc18/models/v6.8_sbc18h2/spectre
Copy the entire directory ‘spectre’ to <Your directory>/Jazz_sbc18h2_v3.1_ADS2011/circuit/models/

The concerned files and directories in this directory are:-

Directories:

NOM

FAST

FASTSLOW

SLOW

SLOWFAST

STAT

X_SIGMA

Files:

cap.scs

circuit.scs

dsv.scs

d_dnw.scs

fet.scs

fuse.scs

global.scs

header.scs

hppnp.scs

hpvar.scs

hpvar_h2.scs

hvres.scs

ind.scs

ind_jit1p4.scs

ldmos.va

lvres.scs

mosvar.scs

n1p8.scs

n3p3_5p0.scs

npn.scs

npn_8v.scs

npn_mod.scs

npn_qpa.scs

npn_rth.scs

one_sigma.scs

one_sigma_h2.scs

p1p8.scs

p3p3_5p0.scs

pc.scs

pnp.scs

pnp_h2.scs

qld.scs

RELEASE_NOTES

res.scs

rfldmos.scs

rnw.scs

rnw.va

salres.scs

schottky.scs

schottky_h2.scs

sections.txt

statistics.scs

twv.scs

IMPORTANT:

- Beginning with ADS 2008 Update 2, the Verilog-A files (underlined in the list above) must be placed in the <design_kit_dir>/veriloga directory. It is recommended to restart ADS after this step.

- If you are working on UNIX, make sure to specify the "-a" to unzip, because the model files are stored in PC format in the zip archive.

## Directions to use the Inductor components (Inductor toolbox)

Inductors apart from the default values may only be used on LINUX and SUN platforms.

Before using the kit in LINUX or SUN environment, 4 variables need to be set before starting ADS.

- setenv RDS_ROOT <Your directory>/Jazz_sbc18h2_v3.1_ADS2011/utilities

- setenv RDS_PT lnx86 (sun5 for SUNOS)

- setenv RDS_CDSWARE <Your directory Jazz_sbc18h2_v3.1_ADS2011/utilities/cdsware

- setenv PERL5 <full path to perl executable>

E.g.

If the installation is done in /home/user1 and perl executable is in /usr/bin/perl, then –

- setenv RDS_ROOT /home/user1/Jazz_sbc18h2_v3.1_ADS2011/utilities

- setenv RDS_PT lnx86 (sun5 for SUNOS)

- setenv RDS_CDSWARE /home/user1/Jazz_sbc18h2_v3.1_ADS2011/utilities/cdsware

- setenv PERL5 /usr/bin/perl

If ADS is already running when you have set these variables, restart ADS.